Managing an Emergency

An Australian (NSW) Perspective

Background:

Events, worldwide, over the past decade have identified differences in management techniques for the control of emergencies.

Simply speaking the management of an emergency falls into three areas:

1.Pre-Emergency:  i.e. identification of hazards, establishment of control measures, training and conduct of simulations.

2.Response: Enactment of Incident control measures, intervention, cooperation of included agencies, management and control of resources.

3.Post: Re-establish/rebuild, investigate, apply system improvements, and deal with litigation and other legal possibilities, implement business continuity measures.

This paper will look at the Australian approach to the first two items.

- Pre Emergency

Hazard Control

All “people at work” are the focus of protection under the Occupational Health & Safety Act (NSW 2000). The object of this Act is “to secure the health, safety and welfare of people at work”.

The object of the Coal Mine Health & Safety Act (NSW 2002) is “to assist in securing the objects of the OH&S Act in relation to coal operations.....”.

The application of the CMH&S Act is to increase the protection of people at work in a coal operation and never results in lesser protection than the overriding OH&S Act.

The operator of a coal mine must prepare a Health and Safety Management System stating how the health and safety of personnel will be protected.

Mining is not allowed without a Health and Safety Management System being implemented.

The Health and Safety Management System must be compiled in consultation with the people at work.

The Health and Safety Management System must be supplied to both the Chief Inspector (Govt appointed) and the Industry Check Inspector (Industry appointed); who may object if they believe it is deficient.

The Health and Safety Management System must be reviewed (at least) on a three yearly basis.

All “Major Hazards” must be identified at operations by the operator and be addressed by the formulation of Major Hazard Management plans. The Minister may declare that an operation is subject to additional Major Hazards, which must be treated in the same way.

The Coal Mine Health & Safety Act also requires an Emergency Management System to be in place to manage any situation not controlled by the health and safety system.

The Emergency Management System must be prepared in consultation with the people that work on the site, identify any risks that could result in an emergency and identify the control structure and key personnel that may be required in an emergency.

Risk Management

All identified hazards are treated using a risk assessment process.

Simply put this involves three simple strategies;

1. Identification of hazards.

2. Analysis and measurement of exposure.

3. Application of control measures to eliminate or reduce the risk.

Under the Coal Mine Health & Safety Regulations (NSW 2006), there is a stated obligation to control risk.

The prime application is to “eliminate”, if this is not possible then the risk must be reduced to as low as reasonably practical by:

- Substitution

- Isolation

- Minimise by engineering principles

- Minimise by administrative means

- Use personal protective equipment

A combination of the above measures is required to reduce the risk to the lowest level practically possible.

Training of the general workforce and elimination of “risk acceptance” is a high priority in Australia.

Risk assessment is considered the norm in all operational activities and is identified as the essential first step.

“On the job” analysis is encouraged in the form of company driven themes to be applied by workers in all situations, an example of such is the SLAM review:

| STOP | Do I understand the task? Am I trained, authorised and competent to do this task? |
| --- | --- |
| LOOK | Is there a documented procedure for this task? What has changed since the last time I did this job? |
| ASSESS | What are the hazards and risks associated with the task/work area? Have I got the right controls in place? |
| MANAGE | Are the controls effective, e.g. guards, barricades, isolation? Am I monitoring the job for new hazards? |

Training

Competency based training and assessment are integral parts of the Australian training system.

Authorisation and certification of competencies determines and verifies a person’s capability to perform a certain task, combined with a rigid reassessment procedure.

Training in emergency response and escape systems is an essential part of the overall safety management system.

An emergency is identified as “when a situation is not controlled by a health and safety management system and there is a threat to life or physical well-being.....”.

The Coal Mines Health and Safety Act require personnel to be trained in the use of all necessary equipment, facilities, systems and procedures related to emergencies.

Simulations

Emergency simulations and exercises are an extremely effective mechanism to test the effectiveness of a system.

These exercises do not necessarily have to impact on the operation of the enterprise and can be conducted to identify individual aspects of the system in isolation. A “closed door” table top exercise that develops depending on the actions and reactions of an Incident Management Team is very effective in determining the operational capabilities of the team, and the system and procedures that they are working with.

Working panel “walk out” trials can easily identify the effectiveness of the training being given to operators in relation to donning of escape apparatus and knowledge of the escape sytem.

Virtual Reality is being used, increasingly, to introduce people to situations that cannot be readily reproduced in mine or simulator applications. The Australian (New South Wales Mines Rescue Service) Virtual Reality application can place students in Spontaneous Combustion, Outburst, Strata Failure (etc) situations that require decision making and problem solving determinations to be made. Results and consequences of these determinations can be identified, repeated and improved upon.

Large scale “whole of mine” simulations are carried out on a regular basis to assess not only the mine’s capabilities but also the interaction of other support agencies.

Continual monitoring and reviewing of the current systems are carried out with improvements being applied.

- Response

Control of an incident is dependent on several factors:

    - Monitoring

    - Trigger Actions

    - Communication

    - Resources

    - Inter Agency Cooperation

Monitoring

The establishment of what is “normal” for a mine can determine at what stage intervention is required.

Systems currently used in Australia include:

Atmosphere – fixed electronic, tube bundle, gas chromatograph, machine mounted, and hand held.

Strata – extensiometers, “tell tales”.

Machinery – vibration, heat, oil.

Ventilation – electronic, in seam measurement.

Airborne Dust – surveys.

Coal Dust – survey and analysis.

By measuring against the pre determined norm, subtle changes can be readily identified and acted upon before they develop into a major problem.

Trigger Actions

The ability of the on-site person to act upon information received is sometimes hindered by the confidence to assess the situation.

The ability for all appropriate levels of management to cause a halt to production, evacuate a mine or section or commit resources to a problem can be addressed by the pre-determination of actions based on the information presented. These triggers can also enable a Rescue Team or Incident Management Team to respond to changes in a timely manner.

The simple “trigger action response plan” below details the actions required by the identified personnel.

Indicates predetermined actions to take place at a mine site.

| Persons Affected | Normal | Level 1 Response | Level 2 Response | Level 3 Response |
| --- | --- | --- | --- | --- |
| TRIGGER GENERAL BODY | CO< 30ppm AND Methane< 1% AND CO2 < 1.25 % AND H2S < 10ppm AND NO2 < 3ppm AND Formaldehyde < 1ppm | Methane concentration 1% to 1.24 % | Methane concentration 1.25% to 2.00% | CO> 30ppm OR Methane> 2.5 % OR CO2 1.25 % OR H2S > 10ppm OR NO2 > 3ppm OR Formaldehyde > 1ppm |
| Operator/Maintainer | - Produce to daily production plan - Shut down non-explosion proof machines at 0.5%. | - Withdraw explosion proof diesel machines to area below 1%. | - Shut down all diesel engines at 1.25% - Shut off electric power at 2%. | - Stop normal operations. - Withdraw to place of safety and await instructions from Team Leader. |
| Control Room Officer | 1 Continue routine monitoring 2 Stop Homotropal belt at 0.5% | 1 Notify relevant Controller of - Nature of alarm - Trend of monitoring points - Barometric trend 2 Organise for a controller to investigate area 3 Record the event on the TARP Status Board 4 Notify underground personnel in the area | 1 Notify relevant Controller of - Nature of alarm - Trend of monitoring points - Barometric trend 2 Record the event on the TARP Status Board 3 Notify VO immediately 4 Notify all underground personnel 5 Stop Homotropal belt at 1.25% | 1 Withdraw men to surface and await instructions from IMT. |

Indicates predetermined mine entry levels for rescue teams.

Because the situation has been identified and the responses quantified, the action (or actions) is the result of an agreed process. Delays in response are eliminated with there being no need for unnecessary discussion or analysis until after the identified actions take place.

Communication

Perhaps the most critical of all issues in relation to the control of an emergency is the communication between the Incident Management Team and the affected personnel. The rapid transfer and assessment of eye witness information will promote the best intervention techniques and the maximisation of resource use.

There are a number of communication systems currently in use in Australia. All current systems rely on a fixed “backbone” to maintain system integrity.

- Leaky Feeder – UG aerial.

- PED – UG or surface aerial.

- Telephone/Pager – fixed line.

The integrity of a communication system is severely challenged as the result of a mine (or section) explosion, fire, inundation or strata failure.

Little has been done in the past to strengthen or protect the required “backbone” in the case of an adverse situation.

The “Mine Safety Technology & Training Commission”, an independent study group reporting to the National Mining Association (US) identified several issues and made recommendations, among these were:

- “Harden” all UG systems and components to withstand an underground occurrence.

- Produce a guideline for “best practice”.

- Develop a hybrid system to allow enhanced wireless systems to operate in conjunction with a reliable, robust leaky feeder.

- Adapt “breadcrumb” technology for use by mines rescue teams.

Valuable advances are being made in regard to “breadcrumb” technology.

These systems consist of nodes that utilize WiFi networking. The nodes are portable with inbuilt power units that can be placed to form a network. Mine communication circuits that use this system can be self repairing, in terms of re-establishing a damaged network, or could quickly be repaired or extended by placement of nodes by rescue personnel. The system is being actively pursued by Australian Mines Rescue.

Closely followed in importance (to communication) is the rapid identification of the location of all personnel.

Several Australian collieries are experimenting with personnel and machinery, location strategies.

Some mines have introduced electronic nodes that are strategically located throughout the colliery to pick up signals from inbuilt “electronic tags” to indicate entry and exit. Electronic tags are generally embedded in cap lamp battery assemblies.

On a more rudimentary scale, a number of sites use a manual tagging system whereby the individual is responsible for “tagging” himself onto a surface located board that indicates which part of the colliery he is going to. The system relies on communication by the individual, and relocation of the tag by others, for subsequent relocations.

The rapid identification of where personnel are situated in the mine allows for more rapid deployment of resources and the ability to focus escape strategies on affected areas in the first instance.

Resources

Rapid access and utilisation of resources is essential for the early control and stabilisation of an event.

Pre-determined allocation of duties, typically in the form of “Duty Cards”, will allow the initial controller of an incident to quickly delegate duties and essential actions to trained personnel in a minimum of time.

An effective Duty Card system should address:

- Site security.

- Manning of communications.

- Notification of essential personnel.

- Notification of required agencies.

- Identification of affected personnel

- Manning of essential services (First Aid, Ventilation, Gas analysis etc)

- Set up of response areas.

Access to response (Mines Rescue) personnel is controlled through the Mines Rescue station in the local area.

Brigadesmen are sourced from the surrounding collieries and become employees of the Mines Rescue Service for the duration of the proceeding actions.

Mines Rescue staff also have access to resources that the colliery may require during an operation e.g.

- Gas analysis expertise

- Gas Chromatograph

- Drillers and equipment

- Inertisation

Inter-Agency Cooperation

There are a number of different agencies and stakeholders with an interest or requirement to be involved in emergency situation in Australia, these include:

- Mine management

- Workforce representatives (local & national)

- Corporate

- Department of Primary Industries (Govt)

- Police

- Ambulance

- Coroner

- Mines Rescue

It is essential that these stakeholders have a working knowledge of each other’s duties and responsibilities in the event of an incident.

The potential for a dysfunctional approach to problem solving is greatly increased when the parties approach is single minded and disjointed in terms of outcomes and correct use of resources.

It has been identified that, while the strict interpretation of the law directs police to take control of the bodies of deceased persons they are neither equipped nor trained to do this in a hostile underground situation.

This has led to a protocol to allow recovery operations to be conducted by Mines Rescue teams in the event of a fatality underground, if the conditions exclude the accompaniment of police officers.

Response Example.

| Time | Event | Actions |
| --- | --- | --- |
| 0530 | In –rush occurrence at mine. | Notification to surface. Activation of Emergency Management System including notification of required personnel and services. |
| 0605 | Mines Rescue, and others, notified. |  |
| 0620 | Mines Rescue arrive on site | Commence data gathering. Preliminary Incident Management Team formed. Risk assessment conducted. Scope of actions considered. Plan and procedures documented and agreed upon. Approval gained. |
| 0645 | Two teams dispatched from surface to affected area. |  |
| 0645 - 0715 | Incident Management Team consolidated. | Incident Management Team (IMT)formed with input from: Mines Rescue Mine Management Police Ambulance Mines Inspectors Site Inspector Risk assessment reviewed, plans and procedures checked for validity. |
| 0715 | FAB established and first team activated. | IMT in direct contact with underground rescue teams to convey and confirm requirements. |
| 0755 | Second Team deployed from FAB | Teams activated to search as per IMT plan. |
| 0815 | Missing personnel located. | Personnel identified as deceased. |
| 0840 | All underground personnel return to the surface. | Body recovery plan established by IMT. |
| 0945 | Body recovery teams dispatched. | Teams formed with appropriate forensic equipment and assistance. |
| 1245 | Bodies brought to mine entries and transferred to ambulances. |  |
| 1310 | Mines Rescue vehicles return to station. |  |
| 1410 | Staff and responders debriefed and counselled. |  |
|  |  | Subsequent actions included: Notes, records and documents gathered and reviewed. Mines Rescue action and response plan and implementation reviewed. Mine Emergency Management System reviewed. Improvements to both systems identified and implemented. |