Fire Fighting in the face 8-91 of the mine Warndt/Luisenthal in 2004

# Ass.of mines,Dr.Ing.Elmar Fuchs

Abstract

On the 27.04.2004 there was an explosion from the abandoned goaf to the tail gate with return air.  Only one miner has been injured.The directly alarmed rescue team has tried to extinguish the open fire in face-gate intersection caused by the explosion. Because of the volume of the fire and the increasing of methane more than 2,5 Vol.% measured by the mobile fire gas analysator the rescue team had to stop the fire fighting measures and to quit the face. The emergency management decided to abandon the fresh and return air gate roads and to inject nitrogen in the abandoned face and tailgates .Thereforthe rescue teams have to build at first leak free air doors in the gate roads near by the face.

To monitor the content of the atmosphere of the abandoned area measurements tubes were laid behind this air doors. With the mobile fire analysator it was possible to measure the content of methane ,CO ,H2,O2,CO2 and higher hydrocarbons.With a special measurement device it was also possible to observe the air temperature. To decrease this temperature liquid nitrogen has been injected through the fresh air barricade.

After 3 weeks the temperature was by 310 C and the content of O2 by

1 Vol.% so that it was sure that the fire has been extinguished. Now  it was possible to reopen the abandoned area without the danger of an another explosion.

To avoid the intake of oxygen in the back tail gate in the next 4 weeks the goaf and the shield support were sealed with phenolic resign foam.

All these fire fighting measurements were observed by the mobile fire gas analysator. After finishing the sealing of the goaf area additional nitrogen with a small rate was injected behind the sealed shield support and the abandoned gate roads. To be sure that there were no other fire places the area behind the roadside pack was observed with a infrared heat picture camera.

Three month after the first explosion and the successful fire fighting it was possible to start again the coal production in the face

8-91 of the mine Warndt/Luisenthal