VIRTUAL REALITY TRAINING

“TOMORROWS TRAINING TODAY”

THE CONCEPT

With the forecast boom in the minerals sector in the late 1990’s, the Board of Coal Services Pty Ltd (CSPL) conducted research into the age and experience levels of the existing workforce. With markets in a subdued state for the past decade and retrenchments and retirements throughout the industry, employment had stagnated. A concerted effort to recruit and prepare a suitable workforce would pose considerable difficulties.                                        					     Primarily:                                                                                                                                               “How to train an inexperienced workforce whilst limiting exposure to hazards?”

In 2000 The Board of CSPL requested the University of New South Wales to conduct a research program into the use of Virtual Reality for training; this was funded by an Australian Coal Association Research Program (ACARP). Once again in 2009 the Board of CSPL has requested the University of Queensland to conduct a research project into “The benefits’ of our Virtual Reality Training (VR) in regards to safety aspects and learning retention”. CSPL in consultation with Mines Rescue (MR) Senior Management embarked on a bold $8.5 Million plan to develop a fully immersive 3D Virtual Reality Training (VR) environment. Simulation scenarios address core competency standards for workers in hazardous environments. This state of the art training tool is designed to give staff practical experience, while reducing risk.

This forms a three pronged philosophy to Competency Based Training

“Our role is to save lives. This technology enables us to teach and retrain safe work procedures to new and experienced miners in a virtual mine before they are exposed to the hazards of a real mine environment”				  Ron Land, Executive Director of Coal Services Pty Ltd

BACKGROUND

In 2002 a grant of $40,000 was issued by an ACARP to the Mines Rescue Service to allow the installation of a flat screened rear projection Virtual Reality [VR] system. It consisted of a 2.3m X 2.3m flat screen controlled from a central control module via touch screen, and incorporated a visualisation hood. The implementation was facilitated by the University of New South Wales Mining Department.

The pilot modules included Roof and Rib Stability, Self Escape and Open Cut Truck Inspection, and was established at Newcastle Mines Rescue Station in the same year.

The second version of Virtual Reality was initiated following a $1.75 million commitment from Coal Services and the Coal Services Health and Safety Trust in 2004. This system consisted of a 160 degree curved screen capability and was our first partnership with iCinema for the development of hardware to run the existing modules.

The current system now utilises a 2 dimensional, 110 degree curved screen environment catering for up to 30 plus participants, and forms just a part of the total VR package.

With the plan now in place, CSPL invested a considerable amount towards upgrading the facilities at the four Mines Rescue Stations, which coverer all mining demographics in the state of New South Wales (NSW), Australia. This was to house not only the existing curved screen technology, but also “I-Dome” and the most advanced fully immersive real world simulator of its kind the “3D Advanced Visualisation and Interactive Environment [AVIE]”.The Newcastle station was the first to be completed in December 2007. The other 3 Rescue stations throughout New South Wales - Singleton, Lithgow and Woonona - came on line over the next 12 months.

IDome						AVIE

The existing modules Roof and Rib Stability, Self Escape and Open Cut Truck Inspection have been improved. Additional modules now include:-

- Isolation

- Hazard Awareness

- Spontaneous Combustion conditions

- Outburst conditions

- Deputies Inspections

.

WHAT IS VIRTUAL REALITY TECHNOLOGY

The total Virtual Reality package consists of three separate systems which all tap into the same software simulation package.

They are:

- Curved Screen

- iDome

- AVIE

Curved Screen

The Curved screen is set up in a theatre configuration and is suited to larger audiences of up to approximately 30 people. A pair of  projectors create a two dimensional image on an 8m X 3.5m 110 degree curved screen, similar to iMax. A third centrally mounted projector allows Powerpoint style presentations to be conducted, which gives the presenter more flexibility when it comes to presentation style

IDome

The iDome is a 3.5m fibreglass hemisphere that was originally developed for musicological applications, and enhances the acoustics in its current application through a fully surround sound audio system. It is a single rear projection onto a parabolic mirror, which is reflected onto the dome screen and provides both a panoramic and spherical view of mining environments.

The iDome system is ideally suited to an individual operator application; however, it can be used for up to 4 people. There are 3 iDomes located at each of the four Mines Rescue facilities.

Advanced Visualisation and Interactive Environment

Utilising the most advanced simulator of its kind, the worlds first Advanced Visualisation and Interactive Environment (AVIE), uses a 120 m2 circular screen to completely surround and immerse the participants.

One master computer is used to coordinate the Image Generators [IGs], and images are projected onto the AVIE screen by 6 pairs of vertically mounted projectors. To achieve the 3D effect, 6 IGs running special software calculate and send separate left-eye and right-eye images to the two projectors in each pair, and the two actual displays onto the same section of the screen. Polarisation filters in front of the projectors, combined with the wearing of polarised glasses when inside the AVIE, filter out the displayed images so that a person only sees the left image through the left eye, and the right image through the right eye – “This gives the user the illusion of seeing 3D."

How does it impact on the individual?

Research suggests that learning retention is greatly enhanced by the use of virtual reality training, as indicated by the graphic below.

“Tell me and I may forget,

Show me and I may remember,

Involve me and I will understand”

With the introduction of the three new VR systems combined with the confidence and experience of the trainers, there has been a steady increase in the usage of Virtual Reality as a training aid.

Personnel Usage Report for Mines Rescue VR Facilities

Historical Data

Q1 2009

NSW TOTAL

Curved	iDome		AVIE		Total

Jan	692		339		743		1774

Feb	784		466		1027		2277

Mar	697		260		757		1784

NEXT GENERATION VIRTUAL REALITY

CSPL Mines Rescue has recently signed a contract with VR Space software development specialists, who will deliver enhanced graphics and flexibility in module design to enable “drag and drop” capability for machinery, personnel, mining equipment and incidents. A single projector has been purchased offering the ability, with the use of 3D glasses, for limited off-site application of VR.

A linked system between the 4 MR facilities in the State will allow real time interaction between training groups immersed in the same scenario but at different locations.

Example of Next Generation Virtual Reality

Whole of Mine Concept

PURPOSE:

This virtual reality module provides a complete underground coal mine, including surface facilities, mine entries, underground roadways, extraction and development systems, men, material and coal transport facilities, and other mine services for the purposes of:

•	Familiarisation of employees and prospective employees with the surface and underground parts of a mine and all infrastructure, equipment and systems.

•	Familiarisation and training with various mining systems, including longwall miner development, brattice ventilation, and auxiliary fan ventilation systems.

•	Training in the operation of longwall equipment using both bi-directional and uni- directional cutting systems.

•	Responding to simulated emergency events such as hazardous atmospheric gas readings, and other emergency conditions.

•	Training candidates for statutory examinations in ventilation, gas management and mining system.

•	Providing an environment and platform for other modules to be developed including hazard awareness, isolation and strata support to be applied for each mining system.

MODULE FEATURES

This module builds on the ventilation and gas management module currently being developed. The whole of the mine module will include the roadways, infrastructure and equipment developed for module 1.

The interaction and other modelling tools developed for the ventilation module will be used and extended to the whole of mine module.

Some details of the mine entries, mine workings, infrastructure, and mine services are described and illustrated in this document and attached drawings.

A major feature of the module is the animation of the longwall equipment. Supports will be seen to lower, advance and set, one at a time, and the shearer will traverse the face cutting in both uni-directional and bi- directional mode. The armoured face conveyor (AFC) will be pushed over as the shearer passes so that a complete cutting cycle is demonstrated.

The module can be operated in a “pre-shift”, and equipment shut down mode, i.e. no equipment operating other than auxiliary fans, and a “during shift” operating mode where all conveyor belts are running and operation of the longwall face is shown.

The mine plan, a gas detector and ventilation measuring device can be called up on screen by the user at any place. The trainer will be able to view the plan and other information on the tablet screen.

PLAN & MINE FEATURES

•	The following plans show the prospective mine roadways and details of mining equipment and infrastructure.

•	The mine layout is designed to show the different mining methods, systems and equipment. Mining methods shown are longwall, continuous miner extraction and continuous miner roadway development. Roadway development is by bolter miner and place changer mining systems. Ventilation is by auxiliary fan and brattice ventilation.

•	There are drifts driven in measures above the seam for transport and conveyor access and these have a different support method.

LONGWALL MINE - ANTITROPAL - U VENT

1	5	10

H	A

MODU

W

OLE

OF

MINE

LE

C

A	B	EAST BLEEDERS

NORTH BLEEDERS

LW 4

DEVELOPMENT

LONGWALL 1	LONGWALL 2

LONGWALL 3	A	B

GOAF

3	3

12 NORTH PANEL

2	2

A	C

3

1

2

A

D	UPCAST SHAFT

COAL BIN

A

12	10

5	1	1

1

10

SERVICES AREA	8

2

E

15

MAIN EAST HEADINGS

Vehicle access door

Return airflow

Intake airflow

3	A

4

D	C

SOUTH DIPS

Page 5

E	C	A

1	5	10

H	A

W

OLE

OF

MINE

MODU

LE

C

A	EAST BLEEDERS

B

NORTH BLEEDERS

LW 4

DEVELOPMENT

Continuous miner with roofbolters

LONGWALL 1	LONGWALL 2

LONGWALL 3	A	B

Continuous miner

with bolters

GOAF

12 NORTH PANEL

Continuous miner

A	C

with bolters

Longwall	3

equipment

2

3

2	Place changer continuous miner

3

1

2

A

D	UPCAST

DH	SHAFT

DH	DH

DH	DH

COAL BIN

DH	DH

A	E

12	5	1	1

10	15

MAIN WEST HEADINGS

Continuous miner with bolters

1

Pump pod

3

SERVICES AREA

2

A

4

D	C

PJB’s parked both sides

8

Diesel bay

Wash down bay

MAIN EAST HEADINGS

Roof bolter

Water

SOUTH DIPS

E

Page 6

C	A	DH

Conveyor drift Portal

Water

Borehole and pump installation

Transport drift Portal

WHOLE OF MINE MODULE

LONGWALL 3

Crib room & CABA cache

89 supports, each 1.75m wide

Double ended ranging drum shearer rides on pan line

PJB

AFC, pans 1.75m in length & 1m wide inside

Stage loader discharges onto panel belt

3

Pod

Outbye end of monorail services connects to fixed line servic2es in roadway- pipe lines, cables etc

Monorail alongside belt suspended from roof - supports services: transformers, load centres, cables, fluid supply hoses, communications, fire line etc. Monorail extends to 400m and “ambushes to 150m — show as 200m length

Stone dust barrier

1

LW3 Loop take up

LW3

Transformers

— LT cables go to monorail

LW3 Drive head

Outbye pump station to supply high pressure fluid to longwall supports via two black 125mm dia hoses

LW3 Drive substation &

belt starter

Belt underpass for travelling road

WHOLE OF MINE MODULE

LW 4 DEVELOPMENT

Continuous miner

fitted with roofbolters	A	B

Pod

5

Shuttle car

Feeder breaker

Pod

Auxiliary fan alongside belt

4

Load centre

Crib room & CABA

cache

Stone dust barrier

3

Pod

Pillars lOOm by 35m centres

PJB

Crem door — belt	2

passes through

LW4 Dev substation

LW4 Dev Loop

take up	l

LW4 Dev Drive head

Belt underpass

for vehicles

LW4 Dev Drive substation &

belt starter

CONFIGURATION TOOLS

The whole of the mine module is to be arranged on a grid system so that the individual grids or locations in the mine can be configured by using the tools menu provided by VR Space.

Configuration tools provided for the ventilation and gas management module include:

•	Gas readings — oxygen, methane, carbon monoxide and carbon dioxide

•	Brattice stopping-partial

•	Brattice stopping-complete

•	Brattice line

•	Remove permanent stopping

•	Place permanent stopping

These tools will apply in any part of the mine.

Configuration tools required for the use of the whole of mine module for mine rescue emergency incidents etc are:

•	Gas readings in any part of the mine, default and configured

•	Air quantity

•	Roof fall

•	Conveyor Belt fire

•	Vehicle fire - in travelling roadways

•	Fire in electrical substation

•	Body — person laying down

•	Person sitting upright

•	Smoke

The advantages for Mines Rescue now are that teams can be presented with a multitude of scenarios through the “whole of mine” concept such as:

- Basic team procedures with timeline for actions

- Communications and communication failure

- Active fire

- Ventilation failure

- Casualties

- Bodies

- Team member injury

- Entrapment

- Critical incident stress within the team

- Aftermath of explosion

- Aftermath of inrush

- Plan reading

INTERACTION & ASSESSMENT

The use of multiple dialogue boxes with questions and answers is considered to be disruptive to the feeling of immersion in the VR environment. Answers to questions on dialogue boxes do not always challenge trainees and often point to the correct answer. It is difficult to develop “quality” incorrect answers.

For these reasons, the use of dialogues will be minimised and a system of audio and visual instructions to trainers implemented. Trainers will communicate instructions and questions to users and trainees.

- System of interaction and assessment needs to be configurable by Mines Rescue staff to provide periodic change to challenge trainees and facilitate the continuous improvement process

- Use of dialogues in new modules is to be minimised and, as far as possible, confined to the introduction of the scenario

- Trainers are to be advised of instructions, questions and answers by audio or visual prompt, and then communicate that information to the user or trainees verbally.

- Information on instructions to trainers, and questions and answers are to be linked to a particular location in the mine, configurable by mines rescue staff

- Visual display tablet (VDT) is to be located in the AVIE with an audio facility

- VDT will have wireless capability so that the trainer can stand away from the console if required to allow a user to operate.

- Primary function of the VDT is to provide advice to the trainer on the trainee’s response to questions through the use of clicka pads

- A clicka pad answer response displayed on the VDT will show the status of trainees answers by number (to identify trainee) and colour (correct/Incorrect)

- VDT may also be used to display questions (but not answers)

- Symbols on the VR screen “I” (information) and “Q” (Question) will appear at locations in the mine which are configurable by Mines Rescue Staff.

- When the symbols appear on the screen, the trainer is advised that information can be accessed. Instructions & questions can be repeated by a trainer.

- The trainer need not access the I or Q information if familiar with the operation of the module.

- The trainer can cause the information to be repeated before moving on if required.

PROVISION FOR FUTURE MODULE DEVELOPMENT

VR Space is required to consider the future extension and expansion of the whole of mine module to include the following:

•Development of isolation, hazard awareness, inspection of production areas, supports scenarios and self escape in any part of the mine.

•Animation of equipment in any working section or other part of the mine.

CONCLUSION

CSPL believe that the key aspects of Virtual Reality training that can benefit and impact your business are:

- Accurate, realistic virtual mine simulations

- Ground breaking interactive 3D technology creating real world experiences

- Simulation of high risk real time scenarios in a safe and controlled environment

- Group or individual training scenarios

- Record of personal assessment and achievement

- High cost equipment remains in production, safely

- High impact experience for maximum learning and retention

- Training has moved to a new level with the introduction of the Virtual Reality program and is a must for any industry developing training packages for hazardous environments.

Resulting in the following benefits

- Improved knowledge and skills reduces injuries and increases productivity

- Workforce has a high level of competence before entering real environment

- Visual interaction from Virtual Reality training modules maintains interest and increases learning retention.

Acknowledgements

CSPL Board of Directors

Bruce Dowsett – CSPL VR Manager

John McKendry – VR Consultant

ICinema – UNSW

VR Space – Software Development