Abstract

New Solution for Using Long-term Respiratory Protective Device in Vehicles

In several German Salt mines, as a consequence of detonations, considerable quantities of carbon dioxide gas (CO2) included in the salt may be discharged. If an eruption of CO2 is indicated on the master display a long-range barrier will be initiated immediately to avoid personel damage and health problems of the employees.

So-called advance and release inspections will be arranged by car to identify the effected underground area and to take further steps to repair or improve the ventilation system. Normally two cars within a distance of about 100 m each driven by one supervisor are used for controlling the extension of CO2. During these missions long-term respiratory protection has to be worn as CO2 is toxic and displaces oxygen (O2).

All types of closed circuit breathing apparatus raise enormous problems when used in vehicles. Due to the size and intended backpack use the mission is very complicated. When placing the device on the passenger’s seat driving is still problematic by virtue of the hose guide. Additionally the user has to overcome the breathing resistance and tolerate inhaled air temperature up to 45‍o C.

The new solution is a compressed air line breathing apparatus supplied from an aluminium case. Inside two breathable gas cylinders delivering fresh air for 90 minutes. These trolleys guaranty easy transportation and safe installation within the car. Two of these trolleys deliver breathable air for 180 minutes.

After arrival at the exclusion zone the new airline device comes into operation. The user gets air via the medium pressure connecting tube to his full face mask. Length of the connecting tube is variable und depends on the mission. In case of a system failure an emergency air supply will be activated automatically. The warning signal of the demand valve starts directly inside the mask

Impairment of driving does hardly occur. Free motion is given and the safety belt can be worn. Breathing resistance is minimal and the breathing air stays comfortably fresh and cool.

The new compressed air line device may also be used for driving scoop trams, loaders and forklift trucks – for underground and surface use, especially for fire fighting in hangars or indoor.

In addition to ergonomical and technical advantages the new airline device is also very interesting regarding economical aspects.